Parameters Setting of A2 Servo Drive

P1-01 = 0x0B (CANopen mode)

P3-00 sets Node ID, the range is 0x01~0x7F

P3-01 sets Baudrate, the range is 150Kbps~1Mbps

P3-09 is the parameter to set SYNC object period, suggestion setting value is 0x5055

After all settings above are done, re-boot the A2 drive

Find Node

Master sends Find Node package

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 0x000 | 0x81 | 0x00 |  |  |  |  |  |  |

A2 replies

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0x700 + node ID | 0x00 |  |  |  |  |  |  |  |

From Id, we can know how many A2 drivers in Bus and what is their node ID respectively

Simple Position Point-to-Point Movement

(Assign target position / arrange target speed / arrange acceleration and deceleration time of velocity profile)

Master to set the control mode to point-to-point movement (Profile Position Mode) [6060h:0 = 1]

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x2F | 0x60 | 0x60 | 0x00 | 0x01 | 0x00 | 0x00 | 0x00 |

Master to set the Target Position as 1000000PUU   [607A h:0 = 1000000[F4240h] ]

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x7A | 0x60 | 0x00 | 0x40 | 0x42 | 0x0F | 0x00 |

Master to set the Profile Velocity as 10000PPS[puu per second]  [6081h:0 = 10000[2710h] ]

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x81 | 0x60 | 0x00 | 0x10 | 0x27 | 0x00 | 0x00 |

Master to set the Profile acceleration as 200ms  [6083h:0 = 200[C8h] ]

Acceleration time is 200ms means motor speed from 0rpm to 3000rpm needs 200ms

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x83 | 0x60 | 0x00 | 0xC8 | 0x00 | 0x00 | 0x00 |

Master to set the Profile deceleration as 300ms  [6084h:0 = 300[12Ch] ]

Deceleration time is 300ms means motor speed from 3000rpm to 0rpm needs 300ms

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x84 | 0x60 | 0x00 | 0x2C | 0x01 | 0x00 | 0x00 |

Master to set the ControlWord to Servo-On the A2 drive  [6040h:0 = 0x0Fh ]

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x2B | 0x40 | 0x60 | 0x00 | 0x0F | 0x00 | 0x00 | 0x00 |

Master to set the ControlWord to trigger the A2 driver to perform absolute position action [6040h:0 = 0x1Fh ]

If set to operate in absolute position, Target Position in setting is actually the target position [absolute position]

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x2B | 0x40 | 0x60 | 0x00 | 0x1F | 0x00 | 0x00 | 0x00 |

Master to set the ControlWord to trigger the A2 driver to perform relative position action  [6040h:0 = 0x5Fh ]

If set to operate in relative position, Target Position in setting is the distance the next action going to travel [relative incremental]

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x2B | 0x40 | 0x60 | 0x00 | 0x5F | 0x00 | 0x00 | 0x00 |

- If going to perform next position command, need to reset the ControlWord to 0x0F and then trigger again

Interpolation Position Movement

Master to set the control mode to interpolation position movement (Interpolated Position Mode) [6060h:0 = 7]

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x2F | 0x60 | 0x60 | 0x00 | 0x07 | 0x00 | 0x00 | 0x00 |

Master to set the communication synchronization period to 4ms   [1006 h:0 = 4] ]

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x06 | 0x10 | 0x00 | 0x04 | 0x00 | 0x00 | 0x00 |

Master to arrange the parameter of PDO: the ID that A2 would receive  [1400h:1 = 0x200 + node ID ]

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x00 | 0x14 | 0x01 | node ID | 0x02 | 0x00 | 0x00 |

Master to arrange the parameter of PDO: the command frequency that A2 would receive  [1400h:2 = 1 ]

A2 will process the position command when receiving SYNC signal from Master for each time

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x2F | 0x00 | 0x14 | 0x02 | 0x01 | 0x00 | 0x00 | 0x00 |

Master to arrange the receiving mapping of PDO: Clean the mapping table

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x2F | 0x00 | 0x16 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |

Master to arrange the receiving mapping of PDO: Adding position command 1

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x00 | 0x16 | 0x01 | 0x10 | 0x01 | 0xC1 | 0x60 |

Master to arrange the receiving mapping of PDO: Adding position command 2

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x00 | 0x16 | 0x02 | 0x10 | 0x02 | 0xC1 | 0x60 |

Master to arrange the receiving mapping of PDO: Adding position difference

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x00 | 0x16 | 0x03 | 0x10 | 0x03 | 0xC1 | 0x60 |

Master to arrange the receiving mapping of PDO: Adding ControlWord

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x23 | 0x00 | 0x16 | 0x04 | 0x10 | 0x00 | 0x40 | 0x60 |

Master to arrange the receiving mapping of PDO: Set the mapping table to 4 sets of Data

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x600 + node ID | 0x2F | 0x00 | 0x16 | 0x00 | 0x04 | 0x00 | 0x00 | 0x00 |

Settings above are for A2 command receiving settings, Master must send one SYNC command and one interpolation command out every 4ms

Assume there are 7 position points, from X-3 to X+3

X-3,  X-2,  X-1,  X,  X+1,  X+2,  X+3

The position difference at X point is [ (X+1) + (X-1) ] / 2

Master to switch A2 to operational mode

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 0x000 | 0x01 | 0x00 |  |  |  |  |  |  |

Then, Master needs send SYNC package and PDO package every 4ms

[Strongly request: the timing of SYNC package sent from Master must be accurate]

Master to send the SYNC package

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0x080 |  |  |  |  |  |  |  |  |

Master to send the PDO package

| Length | Id | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0x200 + node ID | Position Command |  |  |  | Position Difference |  | 0x0F | 0x00 |

[If the data length of position command and position difference was larger than 1 Byte then the data need to be divided in order to fill them into the PDO package.

For example, if the position command is 0x12345678：Byte1 is to fill out 0x78；Byte2 is to fill out 0x56；Byte3 is to fill out 0x34；Byte4 is to fill out 0x12； The fill out format for position difference data is the same.]

Follow the settings above should be able to perform interpolation mode!!!